CS527 — Computer Systems (2026AIM1016)
OpenMP Parallel Programming Lab Report

Tasks 1–5: Correctness, timing, speedup, efficiency, and analysis

Environment: C with GCC and OpenMP (-fopenmp), run in a Linux/WSL terminal. Times below are the measurements recorded during the lab and are machine-specific.

Measurement note: the serial and parallel versions were not always measured with identical warm-up and repetition procedures. Tiny times and unexpected outliers should be treated cautiously; repeat warmed-up measurements and report medians for a more rigorous comparison.

# Summary

Correctness: recorded parallel implementations passed their output checks after fixes. Task 2's intentionally broken shared-sum implementation produced incorrect results with multiple threads.

Reductions safely combine scalar partial results. A scalar max/min reduction does not by itself return the associated index.

Critical sections around every array element cause contention. Manual per-thread extrema and a final merge were much faster in the recorded measurements.

collapse(2) exposes N² iterations instead of N outer-loop iterations, but it does not guarantee faster execution.

Vector addition and matrix addition are sensitive to memory bandwidth. Matrix multiplication is compute-intensive and also sensitive to cache locality.

# Task 1 — Vector Addition

Compute C[i] = A[i] + B[i] using a serial loop and #pragma omp parallel for. Arrays were initialized deterministically, and the serial and parallel outputs were compared element-by-element.

## Correctness verification

for (long i = 0; i < n; i++)
    if (c_serial[i] != c_parallel[i]) { puts("FAIL"); break; }
/* Recorded runs: PASS for every tested N/thread count. */

| N | Serial (s) | Threads | Parallel (s) | Speedup |
| --- | --- | --- | --- | --- |
| 10^5 | 0.000460 | 1 | 0.000183 | 2.508 |
| 10^5 | 0.000460 | 2 | 0.000074 | 6.254 |
| 10^5 | 0.000460 | 4 | 0.000033 | 13.741 |
| 10^5 | 0.000460 | 8 | 0.000019 | 24.236 |
| 10^6 | 0.008137 | 1 | 0.002198 | 3.703 |
| 10^6 | 0.008137 | 2 | 0.000660 | 12.329 |
| 10^6 | 0.008137 | 4 | 0.000455 | 17.866 |
| 10^6 | 0.008137 | 8 | 0.000622 | 13.078 |
| 10^7 | 0.090202 | 1 | 0.013604 | 6.631 |
| 10^7 | 0.090202 | 2 | 0.009756 | 9.246 |
| 10^7 | 0.090202 | 4 | 0.012593 | 7.163 |
| 10^7 | 0.090202 | 8 | 0.015881 | 5.680 |
| 10^8 | 0.693073 | 1 | 0.149368 | 4.640 |
| 10^8 | 0.693073 | 2 | 0.096464 | 7.185 |
| 10^8 | 0.693073 | 4 | 0.072299 | 9.586 |
| 10^8 | 0.693073 | 8 | 0.073045 | 9.488 |

## Questions and observations

Parallelization starts paying off when the amount of work outweighs OpenMP overhead, but the exact crossover depends on hardware and measurement quality. Small arrays finish so quickly that thread startup and scheduling can dominate.

Speedup is not linear with thread count. Vector addition reads two arrays and writes a third, so memory bandwidth becomes a bottleneck. More threads also add runtime overhead and may compete for CPU resources.

The reported small-N speedups are unusually high and likely affected by unequal timing/warm-up effects. At N=10^8, four double arrays consume about 3.2 GB, explaining why larger trials could be killed by memory pressure.

# Task 2 — Sum of Array Elements

Compared serial summation, the intentionally broken shared-sum parallel loop, and reduction(+:sum). The reduction result was checked against the serial sum.

## Correctness verification

long sum = 0;
#pragma omp parallel for reduction(+:sum)
for (long i = 0; i < n; i++) sum += a[i];
/* Compare to sum_serial(a,n): PASS in recorded runs. */

For N=1,000,000 the expected sum was 4,500,000. The broken version produced the following repeated observations:

| Threads | Observed sum (five runs) | Status |
| --- | --- | --- |
| 1 | 4,500,000 | Matches |
| 2 | 2,250,000 | Incorrect |
| 4 | 1,125,000 | Incorrect |
| 8 | 562,500 | Incorrect |

These results happened to equal the correct sum divided by the thread count. This pattern is not guaranteed: a data race is undefined behavior, so different runs, compilers, and machines can produce different results.

| N | Serial (s) | Threads | Reduction (s) | Speedup |
| --- | --- | --- | --- | --- |
| 10^5 | 0.000056 | 1 | 0.000050 | 1.126 |
| 10^5 | 0.000056 | 2 | 0.000026 | 2.187 |
| 10^5 | 0.000056 | 4 | 0.000023 | 2.419 |
| 10^5 | 0.000056 | 8 | 0.001601 | 0.035 |
| 10^6 | 0.000664 | 1 | 0.000527 | 1.259 |
| 10^6 | 0.000664 | 2 | 0.000261 | 2.544 |
| 10^6 | 0.000664 | 4 | 0.000213 | 3.117 |
| 10^6 | 0.000664 | 8 | 0.003978 | 0.167 |
| 10^7 | 0.007377 | 1 | 0.007419 | 0.994 |
| 10^7 | 0.007377 | 2 | 0.004622 | 1.596 |
| 10^7 | 0.007377 | 4 | 0.002863 | 2.576 |
| 10^7 | 0.007377 | 8 | 0.014222 | 0.519 |
| 10^8 | 0.090887 | 1 | 0.072653 | 1.251 |
| 10^8 | 0.090887 | 2 | 0.035553 | 2.556 |
| 10^8 | 0.090887 | 4 | 0.025645 | 3.544 |
| 10^8 | 0.090887 | 8 | 0.024909 | 3.649 |

## Questions and observations

The naive version races because sum += a[i] is a read-modify-write operation. Threads can read the same old value and overwrite each other's updates.

A critical section around each addition would fix correctness but serialize every update and add heavy lock overhead. reduction(+:sum) instead uses thread-local partial sums and combines them at the end.

Parallel reduction may be slower for small arrays because OpenMP overhead exceeds the work. The anomalous 8-thread timings show the need for warm-up and repeated median measurements.

# Task 3 — Maximum/Minimum and Indices

Compared serial extrema, OpenMP max/min value reductions, a critical-section value/index search, and manual per-thread local extrema followed by a serial merge.

## Correctness verification

/* Check both values and indices against serial results. */
if (pmax != smax || pmax_idx != smax_idx ||
    pmin != smin || pmin_idx != smin_idx) puts("FAIL");
else puts("PASS");
/* Final corrected critical and manual implementations: PASS. */

Tie handling: the critical-section condition was updated to choose the smaller index when equal extrema occur. This matches the serial left-to-right scan.

| N | Method | Threads | Time (s) | Check |
| --- | --- | --- | --- | --- |
| 10^5 | Serial | — | 0.000073 | Reference |
| 10^5 | Value reduction | — | 0.061870 | PASS |
| 10^5 | Critical | 1 | 0.008976 | PASS |
| 10^5 | Critical | 2 | 0.007981 | PASS |
| 10^5 | Critical | 4 | 0.036700 | PASS |
| 10^5 | Critical | 8 | 0.047076 | PASS |
| 10^5 | Manual | 1 | 0.000144 | PASS |
| 10^5 | Manual | 2 | 0.000112 | PASS |
| 10^5 | Manual | 4 | 0.005586 | PASS |
| 10^5 | Manual | 8 | 0.017253 | PASS |
| 10^7 | Serial | — | 0.005991 | Reference |
| 10^7 | Value reduction | — | 0.017926 | PASS |
| 10^7 | Critical | 1 | 0.397757 | PASS |
| 10^7 | Critical | 2 | 0.561594 | PASS |
| 10^7 | Critical | 4 | 1.790632 | PASS |
| 10^7 | Critical | 8 | 2.645157 | PASS |
| 10^7 | Manual | 1 | 0.011363 | PASS |
| 10^7 | Manual | 2 | 0.006881 | PASS |
| 10^7 | Manual | 4 | 0.003526 | PASS |
| 10^7 | Manual | 8 | 0.003075 | PASS |
| 10^8 | Serial | — | 0.054231 | Reference |
| 10^8 | Value reduction | — | 0.073364 | PASS |
| 10^8 | Critical | 1 | 3.908499 | PASS |
| 10^8 | Critical | 2 | 7.029884 | PASS |
| 10^8 | Critical | 4 | 17.720274 | PASS |
| 10^8 | Critical | 8 | 26.705772 | PASS |
| 10^8 | Manual | 1 | 0.119767 | PASS |
| 10^8 | Manual | 2 | 0.055628 | PASS |
| 10^8 | Manual | 4 | 0.034217 | PASS |
| 10^8 | Manual | 8 | 0.026925 | PASS |

## Questions and observations

Why not reduction(max:best_idx)? Because the largest index is not necessarily the index of the largest value. For [10,90,30], the maximum is 90 at index 1, but the largest index is 2. Value and index must be kept as a pair.

Manual reduction was much faster than critical sections. The critical approach synchronizes each element's comparison/update, creating lock contention. Manual reduction scans independent chunks and merges only a few local results.

For N=10^8 with eight threads, manual extrema took 0.026925 s versus 26.705772 s for critical sections. Timings are unusually sensitive to environment; the trend is the important observation.

# Task 4 — Matrix Addition

Matrices were stored as flattened contiguous arrays. Element (i,j) is at i*N+j. Both outer-loop and collapse(2) results were checked element-by-element against the serial output.

for (size_t x = 0; x < (size_t)N*N; x++) {
    if (P[x] != S[x]) outer_check = 0;
    if (Q[x] != S[x]) collapse_check = 0;
}
/* Recorded outputs: PASS/PASS at all tested sizes. */

| N | Serial (s) | Threads | Outer (s) | Collapse (s) | Checks |
| --- | --- | --- | --- | --- | --- |
| 8 | ~0.000001 | 1 | 0.000000 | 0.000000 | PASS/PASS |
| 8 | ~0.000001 | 2 | 0.000001 | 0.000001 | PASS/PASS |
| 8 | ~0.000001 | 4 | 0.000001 | 0.000001 | PASS/PASS |
| 8 | ~0.000001 | 8 | 0.000004 | 0.000002 | PASS/PASS |
| 2048 | 0.009572 | 1 | 0.004804 | 0.006367 | PASS/PASS |
| 2048 | 0.009572 | 2 | 0.002111 | 0.002300 | PASS/PASS |
| 2048 | 0.009572 | 4 | 0.001981 | 0.001604 | PASS/PASS |
| 2048 | 0.009572 | 8 | 0.004598 | 0.001512 | PASS/PASS |
| 4096 | 0.057772 | 1 | 0.028980 | 0.029749 | PASS/PASS |
| 4096 | 0.057772 | 2 | 0.009018 | 0.008520 | PASS/PASS |
| 4096 | 0.057772 | 4 | 0.006994 | 0.007810 | PASS/PASS |
| 4096 | 0.057772 | 8 | 0.009643 | 0.006638 | PASS/PASS |

## Questions and observations

For N=8, the outer loop exposes 8 row iterations and collapse(2) exposes 64 element iterations. With at most 8 threads, the outer loop already has one row per thread available, while the operation itself is so small that overhead dominates.

For large N, the outer loop already exposes thousands of rows, enough to keep threads busy. Collapse sometimes wins and sometimes loses; it is not guaranteed to improve performance.

Collapse is most likely to matter when the outer loop has fewer iterations than the thread count or does not expose enough parallel work.

# Task 5 — Naive Matrix Multiplication

The i-j-k algorithm used a local sum for each C[i][j] and wrote that element once. Outer parallelism distributed rows; collapse(2) distributed output-element (i,j) pairs. All recorded parallel output comparisons passed.

for (size_t x = 0; x < (size_t)n*n; x++)
    if (fabs(S[x] - P[x]) > 1e-9) outer_ok = 0;
/* Apply the same tolerance check to collapsed output Q. */
/* Recorded: Outer PASS and Collapse PASS at all N/thread counts. */

| N | Serial (s) | Threads | Outer time | Outer speedup | Outer eff. | Collapse time | Collapse speedup | Collapse eff. | Check |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 0.017281 | 1 | 0.014184 | 1.218 | 1.218 | 0.017065 | 1.013 | 1.013 | PASS/PASS |
| 256 | 0.017281 | 2 | 0.011808 | 1.463 | 0.732 | 0.011878 | 1.455 | 0.727 | PASS/PASS |
| 256 | 0.017281 | 4 | 0.017027 | 1.015 | 0.254 | 0.020946 | 0.825 | 0.206 | PASS/PASS |
| 256 | 0.017281 | 8 | 0.008272 | 2.089 | 0.261 | 0.008219 | 2.103 | 0.263 | PASS/PASS |
| 512 | 0.382945 | 1 | 0.397870 | 0.962 | 0.962 | 0.377836 | 1.014 | 1.014 | PASS/PASS |
| 512 | 0.382945 | 2 | 0.190239 | 2.013 | 1.006 | 0.183330 | 2.089 | 1.044 | PASS/PASS |
| 512 | 0.382945 | 4 | 0.095219 | 4.022 | 1.005 | 0.093121 | 4.112 | 1.028 | PASS/PASS |
| 512 | 0.382945 | 8 | 0.064453 | 5.941 | 0.743 | 0.050790 | 7.540 | 0.942 | PASS/PASS |
| 1024 | 3.474479 | 1 | 3.441870 | 1.009 | 1.009 | 3.521427 | 0.987 | 0.987 | PASS/PASS |
| 1024 | 3.474479 | 2 | 1.850688 | 1.877 | 0.939 | 1.873645 | 1.854 | 0.927 | PASS/PASS |
| 1024 | 3.474479 | 4 | 0.895652 | 3.879 | 0.970 | 0.877361 | 3.960 | 0.990 | PASS/PASS |
| 1024 | 3.474479 | 8 | 0.615529 | 5.645 | 0.706 | 0.593919 | 5.850 | 0.731 | PASS/PASS |

## Questions and observations

Why is it safe to parallelize i? Each thread owns different rows of C. The complete k summation for each C[i][j] stays within the thread responsible for that output element.

Why is parallelizing k unsafe without reduction? Multiple threads could update the same C[i][j] concurrently using +=, creating a read-modify-write race and lost updates.

When does collapse(2) matter? The outer loop provides N iterations; collapse(2) provides N². It matters most if N is small relative to the thread count. In these tests N was already much larger than 8, so differences were modest and inconsistent.

A local sum avoids repeated writes to C during the k loop. For the optional i-k-j ordering, row-major access to B and C is more contiguous in the inner j loop, often improving cache locality; initialize C to zero before accumulating.

# Overall Conclusions

Correctness requires independent output ownership or a suitable reduction/synchronization for shared updates.

A naive shared scalar sum is unsafe; OpenMP reduction is the appropriate fix.

Manual per-thread min/max pairs avoid per-element locks and performed far better than critical sections.

collapse expands the iteration space but is not automatically faster than outer-loop parallelization.

Speedup and efficiency depend on workload size, memory/cache behavior, thread overhead, and system load. Repeated warm-up runs and median timings are recommended.

# Compilation commands

gcc -O2 -Wall -fopenmp task1.c -o task1
gcc -O2 -Wall -fopenmp task2.c -o task2
gcc -O2 -Wall -fopenmp task3.c -o task3
gcc -O2 -Wall -fopenmp task4.c -o task4
gcc -O2 -Wall -fopenmp task5.c -o task5

Example execution: ./task5 1024